Aum Amriteswaryai Namah

LAB Assignment 3

For each question submit the final screenshot.

Q1. In this exercise, you need to use miniedit to create a Network Topology.

Invoke miniedit using the miniedit.py in the examples folder under mininet installation directory.host

- You add two controllers by selecting the control icon. Give the controllers port as 6653 and 6654.

- Then add two OVS switches s1 and s2 by selecting appropriate icon.

- Add two hosts h1 and h2. Then using Link connect h1 with s1 and h2 with s2.

- Right click on Switches and hosts and give them unique IP addresses.

- Connect a link between s1 and s2 and both of the switch should be connected to a controller.

- After building the topology, you may select the following option  “Export Level 2 script”

- You may get more help from the following site.

https://www.brianlinkletter.com/2015/04/how-to-use-miniedit-mininets-graphical-user-interface/

- The saved python script should be modified by adding/modifying the following lines for Ubuntu20.04 users:

  - from mininet.node import Controller, OVSBridge

  - net = Mininet( controller=Controller, switch=OVSBridge)

- Run the pythin script to have a topology with two controllers.

- h1 should be able to ping h2

Q2. In the next experiment, we use two Routers to build a topology with three subnets

Python API given in http://mininet.org/api/annotated.html  to create the topology.

- You need to create two routers with two interfaces. Make sure they have IP address belonging to two different subnets:

  - E.g. Router1 can have  192.68.1.1/24, 10.0.0.3/8, and Router2 can have 10.0.0.2/8, 172.16.0.1/16

  - Router should use static IP routing (It is specified as an option in addNode)

- Add one switch connected to each of the router interfaces

- Add one host connected to each switch. Make sure that host belongs to the same Subnet as the router’s interface.

- Router1 eth2 and Router2 eth1 should be connected. Router1 eth1 should be connected to a host h1 and  Router2 should be connected to Switch s2 and s3 which in turn should be connected to hosts h2 and h3.

- H1 (h1) should have default gateway as Router1 and h2 and h3 should have default gateway as Router2

- Use the skeleton given in myrouter2.py and add more statements required for the steps given above.

- Start the network simulation.

- H1, h2, h3 should be able to ping each other.

Q3. In this question, you will install Pox controller unless it’s already installed on your system.

- Install pox following the instructions in https://noxrepo.github.io/pox-doc/html/

- Then run a L2 learning switch component by running the following command in the directory where pox.py is found. This component makes OpenFlow switches act as a type of L2 learning switch.

./pox.py samples.pretty_log forwarding.l2_learning

- In another window, run mininet with any sample topology and with the option remote controller as pox.

sudo mn –topo=single,4  --controller pox

- Invoke Wireshark in another window or inside mininet and start capturing the packets.

- Do a ping between two hosts. Once you find ping is working, Stop capturing the packets in Wireshark.

- Now locate the following Openflow messages

  - Packet in

  - Echo,

  - Hello and Barrier.